DRAFT FOR DISCUSSION – SUBJECT TO LEGAL, GRANT AND FINANCIAL REVIEW

EXHIBIT E

ACCEPTANCE TESTING STANDARDS

Optical Test Procedures, Pass/Fail Thresholds, and Documentation

by and between

Lumbee Tribe of North Carolina

and

LREMC Technologies, LLC d/b/a RIVR Tech

Version:  Draft v0.1

Version Date:  July 16, 2026

Document Status:  Working draft — not executed; not final legal advice

Program:  Tribal Broadband Connectivity Program (TBCP), National Telecommunications and Information Administration (NTIA)

CONFIDENTIAL – ATTORNEY WORK PRODUCT / SUBJECT TO COMMON-INTEREST PRIVILEGE

DRAFT FOR DISCUSSION – SUBJECT TO LEGAL, GRANT AND FINANCIAL REVIEW

# ARTICLE 1.  SCOPE AND GENERAL REQUIREMENTS

## 1.1  Scope

This Exhibit sets the optical acceptance-testing standards that each route, segment, and span of the Network — and in particular each grant-funded Tribal Asset — must satisfy before it is accepted and placed in service. Acceptance is documented on the Network Acceptance Certificate in Exhibit G.

## 1.2  Test Wavelengths and Method

All fiber shall be tested using bi-directional OTDR at 1310 nm and 1550 nm. Testing shall be performed in both directions on each fiber and the results averaged (bi-directional averaging) to establish the certified splice and span loss.

## 1.3  Instruments and Calibration

OTDR, optical power meter, and light source shall be within current calibration. Calibration certificates: [■ instrument make/model, serial, and calibration dates].

# ARTICLE 2.  TEST PARAMETERS AND PASS/FAIL THRESHOLDS

Each fiber and span shall meet or exceed the following thresholds:

| Test / Parameter | Wavelength(s) | Method | Pass / Fail threshold |
| --- | --- | --- | --- |
| Fusion splice loss | 1310 nm and 1550 nm | bi-directional OTDR (bi-dir avg) | 0.10 dB (average) / 0.30 dB (maximum) per fusion splice |
| Mated connector loss | 1310 nm and 1550 nm | OTDR / insertion loss | 0.50 dB per mated connector pair |
| Connector reflectance | 1310 nm and 1550 nm | OTDR | -40 dB (UPC) / -55 dB (APC) |
| Span / link loss (end-to-end) | 1310 nm and 1550 nm | Power meter (LSPM) + OTDR | consistent with the span-loss budget calculated at 0.35 dB/km at 1310 nm and 0.25 dB/km at 1550 nm, plus splice and connector losses |
| Continuity / polarity | 1310 nm and 1550 nm | Visual / power meter | 100% continuity; correct polarity; no high-loss events |
| Length / distance | 1550 nm | OTDR | [■ design length ± tolerance] |

[TECHNICAL INFORMATION REQUIRED]  Design span-loss budget per route to be computed from as-built lengths, splice count, and connector count and attached to the test record.

# ARTICLE 3.  SPAN-LOSS BUDGET

## 3.1  Calculation

The maximum allowable span loss shall be calculated as the sum of fiber attenuation (consistent with the span-loss budget calculated at 0.35 dB/km at 1310 nm and 0.25 dB/km at 1550 nm, plus splice and connector losses), plus the number of splices times the splice-loss allowance, plus the number of mated connector pairs times the connector-loss allowance, plus a design safety margin of [■ design margin in dB]. Measured span loss shall not exceed the calculated budget.

# ARTICLE 4.  POWER-METER TESTING

## 4.1  Insertion-Loss Test

End-to-end insertion loss shall be measured with a stabilized light source and optical power meter (LSPM) at 1310 nm and 1550 nm, referenced per a one-jumper (or equivalent) reference method, and shall fall within the calculated span-loss budget.

# ARTICLE 5.  TEST DOCUMENTATION

## 5.1  Required Deliverables

The following test records shall be delivered for each accepted segment:

OTDR traces (bi-directional) for every fiber, at both wavelengths, in native and PDF format

Splice-loss and connector-loss tables keyed to the splice diagram

End-to-end insertion-loss (power-meter) results at both wavelengths

Calculated span-loss budget vs. measured loss, with pass/fail per span

Instrument calibration certificates

Fiber assignment / strand mapping keyed to Exhibit C

## 5.2  As-Built and GIS Delivery

As-built drawings and GIS data shall be delivered as a condition of acceptance, including route geometry, splice locations, structure inventory, and strand assignments in the coordinate system identified in Exhibit A. GIS deliverable format: [■ e.g., Esri File Geodatabase / GeoPackage / shapefile + metadata].

[GRANT COMPLIANCE REVIEW REQUIRED]  As-built and GIS deliverables support the equipment/property records required by 2 CFR 200.313 (Equipment — use, management, and disposition) and the asset inventory in Exhibit K.

# ARTICLE 6.  PUNCH LIST, RETESTING, AND ACCEPTANCE

## 6.1  Punch-List Process

Any span, splice, or connector failing a threshold shall be recorded on a punch list identifying the deficiency, location, and corrective action. RIVR Tech shall remediate and retest the affected element.

## 6.2  Retesting

Remediated elements shall be retested using the same method and thresholds. A segment is not accepted until all punch-list items are cleared and the retest passes.

## 6.3  Acceptance Certificate

Upon clearance of the punch list and delivery of the required test documentation, as-built drawings, and GIS data, the segment is accepted by execution of the Network Acceptance Certificate (Exhibit G), which establishes the date placed in service.